DATA VISUALIZATION & ANALYSIS

CAPSTONE PROJECT REPORT

NTPC Power Station

Operational Efficiency Analysis

Research Question Which NTPC power stations have persistent operational problems due to outages, low capacity use, and coal shortages — and how do these problems affect daily power generation?

Dataset: 7,500 records  |  41 Stations  |  20 States  |  2017 to 2026

National Thermal Power Corporation (NTPC) Limited, India

| Project Type DVA Capstone | Domain Energy / Power Sector | Analysis Period 2017 – 2026 |
| --- | --- | --- |

# 1. Abstract

This project looks at how efficiently NTPC (India's largest power company) has been running its power stations over the last 10 years. We used a dataset of 7,500 records covering 41 power stations across 20 states to find out which stations are doing well and which ones are struggling.

We found that the biggest problem stations are gas-based power plants called CCPPs — they're barely running because India doesn't have enough natural gas to supply them. On the other hand, coal-based thermal power stations are mostly working well, with an average of about 84% of their capacity being used.

We also found that the year 2020 was the worst year for NTPC because of COVID-19, which caused major disruptions. But by 2025-2026, things have recovered and NTPC is now performing better than ever. This report covers all these findings in detail and gives suggestions on what NTPC can do to improve further.

# 2. Introduction

## 2.1 About NTPC

NTPC Limited (National Thermal Power Corporation) is India's biggest electricity-generating company. It was set up by the Government of India in 1975, and today it produces about 25% of all the electricity used in the country. NTPC runs power stations in almost every part of India, using different types of fuels like coal, gas, water, and solar energy.

When NTPC runs its power stations efficiently, the whole country benefits — electricity stays cheaper, the grid is more stable, and less fuel is wasted. So studying how well these stations are performing is very important from both an economic and an energy-security point of view.

## 2.2 Why We Did This Study

Even though NTPC is large and well-managed, some of its power stations regularly face problems like:

- Outages — when a unit breaks down or goes offline for repairs

- Low capacity use — when a station produces much less than what it can

- Coal shortage — when there is not enough coal stock to keep running

These problems directly affect how much electricity gets generated every day. Our project tries to figure out which stations face these problems the most, how serious the situation is, and what can be done about it.

## 2.3 Research Question

The main question we are trying to answer is:

"Which NTPC power stations have persistent operational problems due to outages, low capacity utilization, and coal stock constraints — and how do these problems affect daily power generation?"

## 2.4 Objectives

We set the following objectives for this project:

- To study the operational performance of all 41 NTPC stations in the dataset

- To find out which stations are persistently underperforming using a scoring method

- To understand how outages, capacity use, and coal stock are connected

- To look at how performance changed year by year from 2017 to 2026

- To compare gas-based and coal-based stations

- To see which states have the best and worst performing stations

- To suggest practical improvements based on the data

## 2.5 Scope and Limitations

This study is based on 7,500 records of daily operational data from NTPC. The dataset gives us snapshots of performance on specific days — it does not cover every single day for every station. Coal stock data is missing for about 27% of records (mostly gas-based stations, which do not use coal). Maintenance date information is only available for about 16% of records. Because of this, some of our conclusions are based on the available data, which may not be 100% complete.

# 3. About the Dataset

## 3.1 Where Does the Data Come From?

The data comes from NTPC's operational monitoring system. Each row in the dataset represents one observation for a specific power station unit on a particular date. The dataset covers the years 2017 to 2026 and includes stations from 20 different Indian states.

## 3.2 What Variables Are in the Dataset?

Here is a simple description of all the important columns in the data:

| Column Name | What It Means |
| --- | --- |
| Year / Month / Date | The date when this record was captured |
| State / Station Name | Where the station is located and its name |
| Monitored Capacity (MW) | The total installed capacity of the station unit in Megawatts |
| Available Capacity (MW) | How much of that capacity is actually available to generate power on that day |
| Today's Actual Generation (MU) | How much power was actually generated that day (in Million Units) |
| Today's Program Generation (MU) | How much power was planned/scheduled to be generated that day |
| Till Date Actual Generation (MU) | Total actual generation up to that date in the month |
| Coal Stock (days) | How many days of coal supply is available at the current rate of use |
| Capacity Under Outage (MW) | How many MW of capacity is offline due to breakdown or maintenance |

## 3.3 Data Quality Check

Before analyzing the data, we checked for missing values and inconsistencies. Here is what we found:

- Monitored and Available Capacity: Only 26 records (0.3%) have missing values — very clean.

- Coal Stock: Missing for 2,042 records (27.2%). This is mainly because gas-based stations don't use coal, so they have no coal stock data.

- Maintenance Start Dates: Missing for 6,254 records (83.4%). This is expected because maintenance dates are only filled in when a unit is actually under maintenance.

- One record showed a coal stock of 1,517 days — clearly an error. We ignored this outlier in our analysis.

## 3.4 New Columns We Created

To make the analysis easier, we created four new calculated columns:

- Capacity Utilization (%) = Available Capacity / Monitored Capacity x 100. This tells us what percentage of the station's capacity is actually working on a given day.

- Generation Gap = Actual Generation minus Planned Generation. A negative number means the station generated less than it was supposed to.

- Outage Flag = 1 if any capacity is under outage, otherwise 0. This helps us count how often outages happen.

- Low Coal Flag = 1 if coal stock is less than 7 days. The government's Central Electricity Authority (CEA) calls anything below 7 days "Super Critical" — meaning the station is at serious risk of shutting down.

## 3.5 How We Scored Station Inefficiency

To rank stations from most to least inefficient, we created a composite Inefficiency Score. This score combines three factors:

| Factor | Weight | How It's Calculated |
| --- | --- | --- |
| Capacity Underuse | 35% | (100 minus Avg. Capacity Utilization) x 0.35 |
| Outage Frequency | 35% | Outage occurrence % x 0.35 |
| Low Coal Stock Frequency | 30% | % of days with coal < 7 days x 0.30 |

A score of 0 means a perfectly efficient station. A score of 100 means completely inefficient. This formula makes it easy to compare stations fairly, even if they have different types of problems.

# 4. Overall Summary of Findings

## 4.1 Big Picture Numbers

Before diving into individual stations, here is a quick summary of how NTPC is doing overall across all 7,500 records:

| Metric | Overall Value | What's Good | Status |
| --- | --- | --- | --- |
| Avg. Capacity Utilization | 80.7% | Above 85% is ideal | Fair |
| Outage Incidence Rate | 19.2% | Below 10% is good | Needs Work |
| Low Coal Stock (<7 days) | 9.8% | Below 5% is safe | Risky |
| Average Coal Stock | 17.2 days | 21+ days is comfortable | Moderate |

## 4.2 Gas Plants vs. Coal Plants — A Big Difference

One of the most important things we discovered is that there are two very different types of stations in this dataset, and they perform very differently. Gas-based plants (CCPPs) are struggling badly, while coal-based stations are doing quite well.

| Station Type | No. of Records | Avg. Capacity Use | Outage Rate |
| --- | --- | --- | --- |
| Gas-Based (CCPP / Liquid Fuel) | 407 | 22.2% | 92.0% |
| Thermal / Coal-Based | 7,093 | 84.3% | 15.0% |

Why are gas plants doing so poorly? It's not because the plants are broken — it's because India doesn't have enough natural gas to run them. These stations are technically working, but they can't generate power because there's no fuel. This is called the "stranded asset" problem. The 5 worst-performing stations in our entire dataset are all gas-based CCPPs.

# 5. Station-wise Performance Analysis

## 5.1 The 10 Worst Performing Stations

Using our Inefficiency Score formula, we ranked all 41 stations from worst to best. Here are the 10 most problematic ones:

| Station Name | Cap. Use % | Outage Rate % | Low Coal % | Avg. Coal (days) | Avg. Outage (MW) | Score |
| --- | --- | --- | --- | --- | --- | --- |
| ANTA CCPP | 4.3% | 100% | 0% | Gas Plant | 401 MW | 68.5 |
| AURAIYA CCPP | 10.3% | 100% | 0% | Gas Plant | 595 MW | 66.4 |
| DADRI CCPP | 18.7% | 100% | 0% | Gas Plant | 675 MW | 63.5 |
| KAWAS CCPP | 27.6% | 100% | 0% | Gas Plant | 475 MW | 60.4 |
| FARIDABAD CCPP | 29.9% | 100% | 0% | Gas Plant | 303 MW | 59.5 |
| BADARPUR TPS | 12.9% | 68% | 0% | N/A | 79 MW | 54.3 |
| GANDHAR CCPP | 34.2% | 74% | 0% | Gas Plant | 433 MW | 48.9 |
| R. GANDHI CCPP | 31.2% | 69% | 0% | Gas Plant | 248 MW | 48.3 |
| BARAUNI TPS | 36.1% | 61% | 5% | 17.5 days | 93 MW | 45.2 |
| SOLAPUR (Gas) | 34.2% | 56% | 0% | 41.2 days | 371 MW | 42.7 |

## 5.2 The Best Performing Stations

On the other end, here are the stations that are doing excellent work — mostly coal-based plants in eastern India:

| Station Name | Avg. Cap. Use % | Outage Rate % | Avg. Coal (days) | Score |
| --- | --- | --- | --- | --- |
| NORTH KARANPURA TPP | ~95% | Very Low | ~19 | 4.0 |
| SINGRAULI STPS | ~94% | Low | ~17 | 6.8 |
| VINDHYACHAL STPS | ~93% | Low | ~16 | 7.3 |
| RIHAND STPS | ~92% | Low | ~18 | 7.5 |
| BONGAIGAON TPP | ~91% | Low | ~23 | 7.5 |

These stations consistently achieve above 90% capacity use with very low outage rates. They are all coal-based plants located close to coal mines in states like Madhya Pradesh, Uttar Pradesh, Jharkhand, and Assam — which gives them a reliable fuel supply.

# 6. Outage Analysis — The Main Culprit

## 6.1 How Outages Hurt Capacity Utilization

We ran a statistical test (Pearson Correlation) to see how strongly outages are linked to lower capacity utilization. The result was very strong: r = -0.853. This means when outages go up, capacity utilization goes down sharply.

In simple terms: outages are the single biggest reason why stations underperform. Every 1% increase in outage frequency leads to roughly a 0.85% drop in capacity utilization. This is an extremely strong relationship and tells us that fixing outage problems should be the top priority.

| Relationship | Correlation Value |
| --- | --- |
| Outage rate vs. Capacity Utilization | r = -0.853 (Very Strong) |
| Coal Stock vs. Capacity Utilization | r = -0.066 (Weak but significant) |

## 6.2 Which Stations Miss Their Generation Targets the Most?

We also looked at the "Generation Gap" — how much less power a station generates compared to what it was supposed to generate. A large negative gap means the station is regularly underdelivering. Here are the stations with the biggest gaps:

| Station | Actual Generation | Planned Generation | Average Gap (MU) |
| --- | --- | --- | --- |
| SIPAT STPS | 12.3 MU | 2,347.7 MU | -2,335 MU |
| BARH II | 11.4 MU | 2,324.8 MU | -2,313 MU |
| DARLIPALI STPS | 12.1 MU | 2,210.9 MU | -2,199 MU |
| TALCHER STPS | 9.9 MU | 1,910.9 MU | -1,901 MU |
| BARH STPS | 8.9 MU | 1,895.6 MU | -1,887 MU |

Note: The very large gaps in the table above are partly because the 'Planned Generation' column for some records appears to have captured cumulative (monthly) figures instead of daily ones. This is a data quality issue. However, the trend is still clear — stations with frequent outages consistently miss their daily generation targets.

## 6.3 Coal Stock and the Risk of Shutdown

India's Central Electricity Authority (CEA) has set clear guidelines on minimum coal stock levels. Here's how we classify coal stock based on those guidelines:

| Coal Stock Level | Days of Stock | % of Records | Avg. Cap. Use | Risk Level |
| --- | --- | --- | --- | --- |
| Super Critical (Emergency) | Less than 7 days | 9.8% | ~78% | CRITICAL |
| Critical | 7 to 14 days | ~26% | ~80% | HIGH |
| Normal | 14 to 21 days | ~33% | ~83% | MODERATE |
| Comfortable | More than 21 days | ~27% | ~85% | LOW |

# 7. How Has Performance Changed Over the Years?

## 7.1 Year-by-Year Data (2017 to 2026)

Let's look at how NTPC's overall performance changed year by year. We track three main things: average capacity use, outage rate, and average coal stock.

| Year | Avg. Capacity Use (%) | Outage Rate (%) | Avg. Coal Stock (days) | Rating |
| --- | --- | --- | --- | --- |
| 2017 | 84.4% | 16% | Not Available | Good |
| 2018 | 84.8% | 16% | 17.5 days | Good |
| 2019 | 79.5% | 20% | 17.5 days | Fair |
| 2020 | 73.0% | 27% | 23.3 days | Poor |
| 2021 | 76.5% | 23% | 12.5 days | Fair |
| 2022 | 82.0% | 17% | 13.7 days | Good |
| 2023 | 81.9% | 18% | 13.2 days | Good |
| 2024 | 82.4% | 18% | 18.2 days | Good |
| 2025 | 82.6% | 17% | 21.5 days | Good |
| 2026 (Jan-Feb) | 89.3% | 11% | 16.6 days | Excellent |

## 7.2 What the Trends Tell Us

### Phase 1: Stable Start (2017-2018)

In 2017 and 2018, NTPC was performing at its normal baseline — about 84-85% capacity use with 16% outage rate. This gives us a good "normal" level to compare against.

### Phase 2: COVID-19 Disruption (2019-2021)

From 2019 onwards, performance started dropping. The worst year was 2020 when capacity use fell to just 73% and outage rates shot up to 27%. This was clearly because of COVID-19 — demand for electricity dropped, coal supply chains were disrupted, and maintenance work was difficult due to lockdowns. Interestingly, coal stock went UP in 2020 (23.3 days) even though performance was bad — this is because when stations generate less electricity, they burn less coal, so the stock lasts longer. But in 2021, coal stock crashed to only 12.5 days when demand came back suddenly and supply couldn't keep up.

### Phase 3: Recovery (2022-2025)

From 2022, things started getting back to normal. Capacity use climbed back to 82% and outage rates returned to 17-18%. Coal stocks also improved, reaching 21.5 days in 2025, which is the CEA's "comfortable" threshold.

### Phase 4: Best Performance Ever (2026)

The data for January-February 2026 shows NTPC's best-ever numbers: 89.3% capacity use and only 11% outage rate. This is a great sign that NTPC is continuously improving and may be entering a new, higher-performance era.

# 8. Which States Are Performing Best and Worst?

## 8.1 State-by-State Comparison

Where a power station is located in India makes a big difference to how well it performs. States with coal-based plants near coal mines do very well. States with gas-based plants do poorly because of gas shortage. Here is a comparison:

| State | Avg. Cap. Use % | Outage Rate % | Avg. Coal Stock | Overall |
| --- | --- | --- | --- | --- |
| Rajasthan | 4.3% | 100% | No Coal (Gas Plant) | Very Poor |
| Delhi | 12.9% | 68% | N/A | Very Poor |
| Haryana | 29.9% | 100% | No Coal (Gas Plant) | Very Poor |
| Gujarat | 30.7% | 88% | No Coal (Gas Plant) | Poor |
| Karnataka | 62.7% | 37% | 17.9 days | Fair |
| Bihar | 75.8% | 22% | 15.8 days | Fair |
| Uttar Pradesh | 80.5% | 20% | 20.0 days | Good |
| Chhattisgarh | 90.1% | 10% | 11.1 days | Very Good |
| Telangana | 92.0% | 7% | 16.5 days | Excellent |
| Jharkhand | 94.7% | 2% | 19.5 days | Excellent |

## 8.2 Key Geographic Insight

There is a very clear pattern here: states in eastern and central India — like Jharkhand, Telangana, Chhattisgarh, Assam — perform best because they have coal-based power plants that are close to India's coal mining regions. This means they get a steady supply of coal, face fewer outages, and can run at high capacity.

On the other hand, states like Rajasthan, Haryana, and Gujarat perform poorly because their NTPC stations are gas-based CCPPs. These stations aren't struggling because of technical problems — they're struggling because India just doesn't have enough natural gas to run them. This is a policy and infrastructure problem, not an operations problem.

# 9. Key Findings

After analyzing all the data, here are the most important things we found:

### Finding 1: Gas-Based CCPPs Are the Biggest Problem

Five gas-based power plants — ANTA, AURAIYA, DADRI, KAWAS, and FARIDABAD — are barely functioning, with capacity use below 30% and outage rates of 100%. These plants are not broken — they have no gas to run on. This is the largest single source of inefficiency in the NTPC system and cannot be fixed by NTPC alone; it requires national-level gas supply policy reform.

### Finding 2: Outages Are the #1 Driver of Poor Performance

Across all coal-based stations, the main reason why a station underperforms is outages — not coal shortage. The statistical correlation between outages and low capacity use is -0.853, which is extremely strong. This means NTPC should invest heavily in preventing and reducing outages through better maintenance practices.

### Finding 3: Eastern States Are NTPC's Backbone

Jharkhand, Telangana, Chhattisgarh, and Assam consistently show the best performance. Their coal-based stations achieve 90-95% capacity use with very low outage rates. These stations should be used as a benchmark for others to follow.

### Finding 4: COVID-19 Caused the Worst Year (2020)

The year 2020 was NTPC's worst in the decade — 73% capacity use and 27% outage rate. This was directly caused by COVID-19 disruptions. The coal crisis in 2021 (where stock fell to 12.5 days) was also a knock-on effect of the pandemic. The good news is NTPC has fully recovered from this by 2022.

### Finding 5: 2026 is the Best Year on Record

Data from January-February 2026 shows 89.3% capacity use and only 11% outage rate — the best figures in the entire dataset. This suggests that NTPC's ongoing modernization and improved operations are starting to show results.

### Finding 6: BARAUNI TPS Needs Urgent Attention

Among coal-based stations, BARAUNI TPS in Bihar stands out as the most problematic — it has 61% outage incidence across 191 records. Unlike the gas plants, BARAUNI's problems are genuine operational and maintenance issues that NTPC can and should fix directly.

# 10. Our Recommendations

Based on our analysis, here are the steps we suggest NTPC and the government should take to improve performance:

## 10.1 Things to Do Right Away (0-6 Months)

- Set a Minimum Coal Stock Rule: Make it mandatory for all coal-based NTPC stations to always keep at least 21 days of coal stock (the CEA's comfortable level). If stock falls below 14 days, send out automated alerts. Below 7 days, start emergency buying.

- Build a Live Dashboard for Operations: Create a real-time screen in each regional operations center showing capacity use, outage status, and coal stock for every station. This will help managers spot problems early before they become serious.

- Fix BARAUNI TPS First: BARAUNI has a 61% outage rate across 191 records — that's the worst among coal-based stations. Start a full maintenance audit there immediately and allocate budget for repairs.

## 10.2 Medium-Term Actions (6-18 Months)

- Convert Stranded Gas Plants to Peaking/Renewable Support: The 5 gas CCPPs (ANTA, AURAIYA, DADRI, KAWAS, FARIDABAD) have huge installed capacity sitting idle. Consider repurposing them as backup/peaking plants that kick in when solar and wind energy is not available. This way, the infrastructure investment is not wasted.

- Use Predictive Maintenance at High-Outage Stations: For stations with 20-60% outage rates (like KUDGI, TANDA, BARAUNI), use sensors and machine learning to predict equipment failure before it happens. This can reduce unplanned outages by up to 30-40%.

- Improve Coal Forecasting: Connect NTPC's planning systems with Coal India's dispatch data and railway tracking so that each station can predict coal arrivals 7-14 days in advance. This removes uncertainty in coal stock management.

## 10.3 Long-Term Strategic Actions (18+ Months)

- Make the Inefficiency Score a Regular KPI: Use the composite Inefficiency Score developed in this project as an official quarterly review metric. Tie it to station management accountability and capital allocation decisions.

- Retire the Oldest Inefficient Stations: Stations like BADARPUR TPS (already retired) and aging units elsewhere with scores above 40 should be evaluated for early retirement. The money saved can be reinvested in modern, ultra-efficient plants.

- Lobby for Gas Supply Infrastructure: NTPC should work with the Ministry of Power and Gas to push for more LNG import terminals or gas pipeline expansion. Without a reliable gas supply, the CCPP stations will never be efficient regardless of how well they are managed.

# 11. Conclusion

This project set out to find which NTPC power stations are struggling the most with operational inefficiency — and why. After analyzing 7,500 records covering 41 stations across 10 years, we now have a very clear picture.

The most important finding is that NTPC has two very different kinds of operational problems. First, there is the gas plant problem — five CCPPs in northern and western India are barely running because India does not have enough natural gas to supply them. This is not something NTPC can fix on its own; it requires government-level policy changes. Second, there is the coal plant challenge — most coal-based stations are doing well, but a few older stations like BARAUNI TPS have persistent maintenance problems that drive high outage rates.

Across the board, outages are the biggest enemy of efficiency. When a unit goes offline — whether for planned maintenance or an unexpected breakdown — it has a massive impact on how much power the station can generate. This is confirmed by a very strong statistical correlation of -0.853. The lesson here is clear: if NTPC wants to improve, it needs to reduce outage frequency above everything else.

The good news is that NTPC is heading in the right direction. The 2026 data is the best in 10 years, with 89.3% capacity use and only 11% outage rate. If NTPC follows through on modernization, better maintenance, and coal supply chain improvements, it has a real chance of consistently hitting 90%+ efficiency across its coal fleet — which would be a major achievement for India's energy security.

This capstone project has given us valuable practice in real-world data analysis — from cleaning and exploring a large dataset to building a composite scoring model and drawing actionable insights. We hope these findings are useful for understanding how data can drive better decisions in the energy sector.

End of Report

DVA Capstone  |  NTPC Operational Efficiency Analysis  |  Data: 2017–2026

# Appendix: Glossary of Terms

Here is a quick reference guide for terms used in this report:

| Term | What It Means |
| --- | --- |
| NTPC | National Thermal Power Corporation — India's largest power generation company, owned by the central government. |
| MW (Megawatt) | A unit of power capacity. 1 MW can power about 800-1,000 homes. Most NTPC units range from 200 MW to 800 MW. |
| MU (Million Units) | A unit of electricity generation equal to 1,000 Megawatt-hours (MWh). This is what gets reported as actual daily generation. |
| Capacity Utilization (%) | Percentage of a station's total capacity that is actually available and working on a given day. Higher is better. |
| CCPP | Combined Cycle Power Plant — a gas-based plant that uses both gas and steam turbines. More efficient in theory, but dependent on gas supply. |
| STPS | Super Thermal Power Station — NTPC's name for its largest coal power plants (usually above 1,000 MW total capacity). |
| Outage | When part or all of a power plant unit is offline and not generating electricity, either due to breakdown or scheduled maintenance. |
| Super Critical Stock | When a station has less than 7 days of coal left. The CEA classifies this as an emergency level. |
| Inefficiency Score | A score created for this project that combines capacity underuse, outage frequency, and low coal stock into a single number. Higher score = more inefficient. |
| Pearson Correlation (r) | A statistical measure of how strongly two things are related. Values close to -1 or +1 mean a very strong relationship. Close to 0 means weak relationship. |
| CEA | Central Electricity Authority — the government body in India that sets guidelines and standards for the power sector, including minimum coal stock levels. |
| Stranded Asset | A power plant that has been built and paid for but cannot be run because the fuel it needs is not available. Most NTPC CCPPs are stranded assets. |